MEMO

To:		Jim Titus

EPA

From:		Jue Wang, Kim Balassiano

ICF Consulting, Inc.

Date:		December 11, 2002

Subject: 	Results from Benchmarking Interpolation Methods for Ocean City, MD

The attached graphics will hopefully be useful in identifying the attributes and limitations of each interpolation method.  The initial finding is that TOPOGRID, the method that has been implemented to date, does seem to be the most successful – across the board – at modeling terrain.  Some methods, such as IDW and TIN, are better at retaining input values, however, they are generally poorer at trend analysis and interpreting hills and valleys, and therefore result in plateau creation.  Splining appears to have the most difficulty in trend analysis and the results include artificial hills based on elevation values well outside the range of input elevation values.

A graphic depicting the results from another TOPOGRID approach, which is to cut the modeling area into 3 parts (greater than 5 ft, between 5 ft and the upper limit of the tidal wetland, all tidal wetland) has also been included below.  Approaches such as this are designed in response to the apparent shortcomings of the interpolation method of choice.  It is likely that such creative approaches, in addition to hybrid approaches as implemented by the USGS (which uses IDW in combination with splining), can be useful in perfecting any output model.  However, given the time frame and budget, extensive research, testing, and benchmarking of different inputs for the various parameters, much less, writing a program that operates outside the given parameters of off-the-shelf ArcInfo software, are beyond the scope of this project.

Statistics showing the area within each half-foot increment between –10 and 20 ft have been collected for each interpolation method and are shown below. Following the statistics, the first five graphics depict results from IDW, splining, TIN, TOPOGRID, and corrected TOPOGRID (using the 3-part approach).  The second series of graphics depict the effect of the first four techniques on bogus input contours to demonstrate the differences more precisely.  Also included is a histogram that shows the values along the transect which has been drawn in exactly the same place across each graphic depicting the bogus data.  Discussion of the graphics is available below.

| Vertical units in feet, and area in square kilometers. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ELEVATIONS |  | Area in Elev. Class by Interpolation Method |  |  |  |  |
| > | <= | TOPO | SPLN | TIN | IDW |  |
| -10 | -9.5 | 0.00 | 0.06 | 0.00 | 0.00 |  |
| -9.5 | -9 | 0.00 | 0.07 | 0.00 | 0.00 |  |
| -9 | -8.5 | 0.00 | 0.08 | 0.00 | 0.00 |  |
| -8.5 | -8 | 0.00 | 0.08 | 0.00 | 0.00 |  |
| -8 | -7.5 | 0.00 | 0.12 | 0.00 | 0.00 |  |
| -7.5 | -7 | 0.00 | 0.10 | 0.00 | 0.00 |  |
| -7 | -6.5 | 0.00 | 0.10 | 0.00 | 0.00 |  |
| -6.5 | -6 | 0.00 | 0.12 | 0.00 | 0.00 |  |
| -6 | -5.5 | 0.00 | 0.11 | 0.00 | 0.00 |  |
| -5.5 | -5 | 0.00 | 0.16 | 0.00 | 0.00 |  |
| -5 | -4.5 | 0.00 | 0.14 | 0.00 | 0.00 |  |
| -4.5 | -4 | 0.00 | 0.16 | 0.00 | 0.00 |  |
| -4 | -3.5 | 0.00 | 0.20 | 0.00 | 0.00 |  |
| -3.5 | -3 | 0.00 | 0.23 | 0.00 | 0.00 |  |
| -3 | -2.5 | 0.00 | 0.28 | 0.00 | 0.00 |  |
| -2.5 | -2 | 0.00 | 0.34 | 0.00 | 0.00 |  |
| -2 | -1.5 | 0.00 | 0.38 | 0.00 | 0.00 |  |
| -1.5 | -1 | 0.02 | 0.46 | 0.00 | 0.00 |  |
| -1 | -0.5 | 0.16 | 0.65 | 0.00 | 0.00 |  |
| -0.5 | 0 | 0.77 | 0.92 | 0.00 | 0.00 |  |
| 0 | 0.5 | 4.78 | 2.58 | 1.46 | 0.68 |  |
| 0.5 | 1 | 29.54 | 24.73 | 37.68 | 23.15 | Mean SHW is somewhere around here. |
| 1 | 1.5 | 3.54 | 2.76 | 1.97 | 3.85 |  |
| 1.5 | 2 | 2.50 | 1.76 | 1.72 | 3.13 |  |
| 2 | 2.5 | 2.01 | 1.50 | 1.80 | 2.82 |  |
| 2.5 | 3 | 1.81 | 1.45 | 1.83 | 2.58 |  |
| 3 | 3.5 | 1.96 | 1.51 | 1.75 | 2.28 |  |
| 3.5 | 4 | 2.26 | 1.64 | 1.81 | 2.84 |  |
| 4 | 4.5 | 3.03 | 2.14 | 1.81 | 4.12 |  |
| 4.5 | 5 | 9.17 | 8.87 | 18.48 | 16.54 | Notice peaks near contour lines. |
| 5 | 5.5 | 9.21 | 7.13 | 2.19 | 4.85 |  |
| 5.5 | 6 | 3.63 | 2.73 | 2.21 | 2.53 |  |
| 6 | 6.5 | 2.80 | 2.05 | 2.13 | 1.78 |  |
| 6.5 | 7 | 2.33 | 1.99 | 2.18 | 1.58 |  |
| 7 | 7.5 | 2.23 | 1.86 | 2.18 | 1.49 |  |
| 7.5 | 8 | 2.22 | 1.82 | 2.13 | 1.64 |  |
| 8 | 8.5 | 2.37 | 1.84 | 2.18 | 1.82 |  |
| 8.5 | 9 | 2.89 | 2.05 | 2.15 | 2.40 |  |
| 9 | 9.5 | 3.68 | 2.65 | 2.15 | 3.23 |  |
| 9.5 | 10 | 15.31 | 27.65 | 35.51 | 34.54 | Notice peaks near contour lines. |
| 10 | 10.5 | 16.08 | 7.58 | 1.19 | 2.59 |  |
| 10.5 | 11 | 3.11 | 2.27 | 1.19 | 1.39 |  |
| 11 | 11.5 | 1.85 | 1.59 | 1.22 | 1.08 |  |
| 11.5 | 12 | 1.36 | 1.35 | 1.15 | 0.87 |  |
| 12 | 12.5 | 1.12 | 1.17 | 1.15 | 0.85 |  |
| 12.5 | 13 | 1.05 | 1.12 | 1.15 | 0.85 |  |
| 13 | 13.5 | 1.17 | 1.15 | 1.11 | 0.95 |  |
| 13.5 | 14 | 1.28 | 1.21 | 1.06 | 1.15 |  |
| 14 | 14.5 | 1.61 | 1.39 | 1.08 | 1.39 |  |
| 14.5 | 15 | 3.96 | 6.37 | 9.51 | 8.58 | Fewer 15 ft lines in input data. |
| 15 | 15.5 | 4.40 | 3.30 | 1.00 | 2.20 |  |
| 15.5 | 16 | 1.54 | 1.40 | 0.97 | 1.33 |  |
| 16 | 16.5 | 1.19 | 1.20 | 0.94 | 0.92 |  |
| 16.5 | 17 | 1.01 | 1.08 | 0.94 | 0.79 |  |
| 17 | 17.5 | 0.97 | 0.99 | 0.93 | 0.76 |  |
| 17.5 | 18 | 1.08 | 0.95 | 0.90 | 0.83 |  |
| 18 | 18.5 | 1.21 | 1.02 | 0.89 | 0.85 |  |
| 18.5 | 19 | 1.51 | 1.12 | 0.91 | 1.04 |  |
| 19 | 19.5 | 2.12 | 1.46 | 0.83 | 1.42 |  |
| 19.5 | 20 | 6.58 | 11.99 | 15.40 | 15.37 | Notice peaks near contour lines. |

Notes on 5 graphics above:

- Notice how in this area all interpolation methods create plateaus where there is only a 10 ft contour available.  Only in the TOPOGRID process does this problem somewhat resolve itself and trend analysis identifies hills and valleys

- Another example of interpolation methods described in A.

- Example of how splining can cause artificial hills (and depressions) where contours are close together and contour values indicate a steep rise.  Splining tends to exaggerate the slope and resultant interpolation.

Notes on 4 graphics above:

IDW

1- No sense of hill or trend, instead a plateau is created.

2- No sense of valley or trend between this area and area below it.

3- Even spread of values on both sides of contour.

SPLINE

1- Artificial hill caused by extremely exaggerated trend analysis near unevenly distributed contour lines.

2- No sense of hill or trend, instead a plateau is created.

3- Good interpolation of contours.

4- Artificial depression caused by closely-located but somewhat steep contours.

TIN

1- Reliable interpolation where contours are evenly spaced.

2- No sense of valley or trend between this area and area below it.

3- No sense of hill or trend, instead a plateau is created.

TOPOGRID

1- Artificial depression caused by exaggerated trend analysis near unevenly distributed contour lines.

2- Good trend analysis identifies hills (and valleys).

3- Good trend analysis identifies hills (and valleys).

4- No artificial depressions in areas where neighboring lines are evenly distributed.

Transects from Above 4 Graphics

(A to B as you move left to right on graph)